电 子 科 技 大 学

UNIVERSITY OF ELECTRONIC SCIENCE AND TECHNOLOGY OF CHINA

硕士学位论文

MASTER THESIS

（电子科技大学图标）

论文题目

学科专业

学     号

作者姓名

指导教师

分类号                                   密级

UDC注1

学  位  论  文

（题名和副题名）

（作者姓名）

指导教师

（姓名、职称、单位名称）

申请学位级别              学科专业

提交论文日期          论文答辩日期

学位授予单位和日期  电子科技大学             年   月   日

答辩委员会主席

评阅人

注1：注明《国际十进分类法UDC》的类号。

Calderόn Technique Based Integral Equation Methods in Computational Electromagnetics

A Master Thesis Submitted to

University of Electronic Science and Technology of China

Major:       Electromagnetic Field and Microwave

Technology

Author:                    XXX

Supervisor:               Prof. Li Si

School:        School of Electronic Engineering

独创性声明

本人声明所呈交的学位论文是本人在导师指导下进行的研究工作及取得的研究成果。据我所知，除了文中特别加以标注和致谢的地方外，论文中不包含其他人已经发表或撰写过的研究成果，也不包含为获得电子科技大学或其它教育机构的学位或证书而使用过的材料。与我一同工作的同志对本研究所做的任何贡献均已在论文中作了明确的说明并表示谢意。

作者签名：                       日期：    年   月   日

论文使用授权

本学位论文作者完全了解电子科技大学有关保留、使用学位论文的规定，有权保留并向国家有关部门或机构送交论文的复印件和磁盘，允许论文被查阅和借阅。本人授权电子科技大学可以将学位论文的全部或部分内容编入有关数据库进行检索，可以采用影印、缩印或扫描等复制手段保存、汇编学位论文。

（保密的学位论文在解密后应遵守此规定）

作者签名：                   导师签名：

日期：     年   月   日

摘  要

本文基于电磁理论中的Calderόn关系与Calderόn恒等式所揭示的不同积分算子之间的关系，系统地研究了Calderόn预条件技术及其在计算电磁学积分方程方法中的应用。研究内容全面覆盖了求解理想导电体目标和均匀或分层均匀介质目标电磁散射与辐射问题的积分方程中的Calderόn预条件技术。在导体积分方程方面，研究了电场积分方程在中频，低频，以及高频区的Calderόn预处理方法。在介质积分方程方面，则研究了PMCHWT积分方程的Calderόn预处理方法，和N-Müller积分方程的Calderόn技术。本文对金属问题中的第二类Fredholm积分方程和介质问题中的第二类Fredholm积分方的精度改善进行了深入详尽的研究。

……

关键词：电磁散射，面积分方程方法，Calderόn预条件方法，数值计算精度，第二类Fredholm积分方程

ABSTRACT

Revealed by the Calderόn relation and the Calderόn identities in electromagnetic theory, the properties and relation of different integral operators in the computational electromagnetics (CEM) are utilized to construct the Calderόn preconditioning techniques, which are applied in the integral-equation-based methods in this thesis. A thorough and systematic research has been accomplished to cover the Calderόn preconditioning techniques for the perfect electric conductor (PEC) and the dielectric cases. For the PEC case, the Calderόn preconditioning for the electric-field integral equation (EFIE) at mid, low, and high frequencies are constructed and studied. For the dielectric cases, the Calderόn preconditioning for the Poggio-Miller-Chang-Harrington-Wu-Tsai (PMCHWT) integral equation are investigated, and the Calderόn technique for the N-Müller integral equation is developed. Moreover, the accuracy improving technique for the second-kind Fredholm integral equation for both PEC and dielectric cases is also studied in this thesis.

…

Keywords: Electromagnetic scattering and radiation, surface-integral-equation-based Methods, Calderόn preconditioning methods, numerical accuracy, Fred-

holm integral equations of the second kind

Contents

Chapter 1 Introduction	1

1.1 Research Background and Significance	1

1.2 State of Arts	1

1.3 Contents and Innovations of the Thesis	2

1.4 Outline of the Thesis	2

Chapter 2 Theoretical Basics	3

2.1 Integral Equations in Electromagnetics	3

2.2 270 MHz Plan Wave Excitation	3

2.3 The Solution of Integral Equations in Electromagnetics	4

2.3.1 General Principle of the Method of Moments	4

2.3.2 Geometrical Modeling and Discretization of Object	4

2.3.2.1 Planar Triangular Model	4

2.3.2.2 Curvilinear Triangular Model	4

2.3.3 The Choice of Basis Functions	5

2.3.3.1 Planar RWG Basis Functions	5

2.3.3.2 Curvilinear RWG Basis Functions	6

2.3.4 The Solution of Matrix Equations	6

2.3.4.1 Direct Algorithms	6

2.3.4.2 Iterative Algorithms	6

2.4 Conclusion	6

Chapter 3 Calderόn Preconditioner at Mid Frequencies	7

3.1 Introduction	7

3.2 Calderόn Relation and Calderόn Identities	7

3.3 Calderόn Preconditioner at Mid Frequencies	7

3.4 Numerical Examples	7

3.5 Conclusion	7

Chapter 4 Calderόn Preconditioning Technique for N-Müller	8

4.1 Introduction	8

4.2 N-Müller Integral Equations	8

4.3 The Derivation of N-Müller Equations	8

4.4 The Discretization of N-Müller Equations	8

4.5 Numerical Examples	8

4.6 Conclusion	8

Chapter 5 Conclusions	9

5.1 Concluding Remarks	9

5.2 Future Work	9

Acknowledgements	10

References	11

Research Results Obtained During the Study for Master Degree	12

Chapter 1 Introduction

- 1.1 Research Background and Significance

Integral-equation-based numerical methods combined with fast algorithms are capable of solving electromagnetic problems of complex structures and material properties with a good accuracy and a high efficiency. They are widely used in a variety of engineering applications, such as the efficient analysis of three dimensional radar scattering problems, the simulation of the input impedance and the radiation properties of antenna systems, the calculation of the input response and the transmission efficiency of microwave circuits, the evaluation of the electromagnetic interference (EMI) between complex electromagnetic systems, and the computer aided electromagnetic compatibility (EMC) designs. The versatility, capability, accuracy and efficiency of the integral-equation-based methods have made them an important and cost effective approach in the analysis and design of electromagnetic problems and applications.

…

- 1.2 State of Arts

From the 1960s, the numerical methods of electromagnetic analysis have been fast developed because of their versatility and flexibility. Many well-known numerical methods have been introduced during that time, including the finite element method (FEM) [1] and the finite difference time domain method (FDTD) [2,3], which are based on the solution to the Maxwell’s equations in differential form, and the method of moments (MoM) [2, 4-6], which is based on the solution to the Maxwell’s equations in integral form. Especially from 1990s, with the fast developments of high performance computing systems, the theories and methods of computational electromagnetics have been advanced dramatically. The increases of the clock speed and the memory size of computer systems and the developments of highly efficient electromagnetic computing algorithms make the numerical methods capable of solving  electromagnetic engineering problems.

…

- 1.3 Contents and Innovations of the Thesis

Based on the Calderόn relation and the Calderόn identities, this thesis has developed several Calderόn preconditioning techniques and investigated their applications in the integral-equation-based computational electromagnetic methods. The research content has covered the Calderόn preconditioning techniques for the perfect electric conductor (PEC) and dielectric cases. For the PEC case, the Calderόn preconditions at mid, low, and high frequencies are investigated. For the dielectric case, the Calderόn preconditioning techniques for the PMCHWT and N-Müller integral equations are developed. The numerical accuracy of the second-kind Fredholm integral equations are investigated and improved in this thesis.

…

- 1.4 Outline of the Thesis

This thesis is organized as follows.

…

- Chapter 2 Theoretical Basics

In this chapter, the general methods of constructing the commonly used integral equations in electromagnetics are introduced based on the surface equivalence principle and the volume equivalence principle.

…

- 2.1 Integral Equations in Electromagnetics

In the integral-equation-based computational electromagnetic methods, the unknown functions in the electromagnetic problems such as the scattering or radiation fields are modeled in terms of the equivalent surface or volume electric/magnetic sources by applying the surface or volume equivalence principles, respectively.

…

- 2.2 270 MHz Plan Wave Excitation

In order to investigate the its performance in handling electrically very large problems with over one million unknowns, the same numerical example is repeated again by increasing the frequency to 270 MHz, and keeping the incident angle and polarization of the plane wave unchanged. To have a better insight, the memory consumption and CPU time requirements of the EFIE, the CP-CFIE(0.8), and the CP-AEFIE algorithms are given in Table 2-1.

Table 2-1 Comparison of Computational Data of Different Algorithms

|  | Total Memory (Mb) | CPU Time |  |  |
| --- | --- | --- | --- | --- |
|  |  | Setup (h) | Solution Time |  |
|  |  |  | Iter. (m) | Tol. (h) |
| EFIE | 3215.84 | 1.14 | 3.18 | >63 |
| CP-CFIE(0.8) | 6386.12 | 7.84 | 7.04 | 27.69 |
| CP-AEFIE | 5750.43 | 6.71 | 7.47 | 19.05 |

All the calculations are carried out on a HP Z400 workstation with a Fedora 10 operating system.

…

- 2.3 The Solution of Integral Equations in Electromagnetics

- 2.3.1 General Principle of the Method of Moments

The integral equations constructed in the preceding section can be solved with adequate numerical methods. One of the most commonly used methods in solving integral equations is the method of moments (MoM) introduced by R. F. Harrington in 1968[5]. The general principle and key points of MoM will be reviewed in this section.

…

- 2.3.2 Geometrical Modeling and Discretization of Object

From the description in the preceding section, it is clear that in order to solve for the unknown equivalent electromagnetic currents defined on the surface or in the volume of an obstruction, the definition domain of the unknown currents, which is the geometry, needs to be described mathematically. This is the so-called geometrical modeling. In computational electromagnetics, geometrical modeling is the basic of electromagnetic modeling and numerical calculation, and its quality will affect the accuracy of the numerical solution directly.

…

- 2.3.2.1 Planar Triangular Model

The simplest and most commonly used element in the geometrical modeling is the planar triangle, which is defined by its three vertices (nodes).

…

- 2.3.2.2 Curvilinear Triangular Model

The curved surface of an object can be better modeled with curvilinear triangular elements which are the second-order curved surfaces. A curvilinear triangle can be defined by six nodes, three of which are the vertices of the triangle, the other three are the midpoints of three curved edges. Shown in Figure 2-1 is the sketch of a curvilinear triangular element.

The curved surface of an object can be better modeled with curvilinear triangular elements which are the second-order curved surfaces. A curvilinear triangle can be defined by six nodes, three of which are the vertices of the triangle, the other three are the midpoints of three curved edges. Shown in Figure 2-1 is the sketch of a curvilinear triangular element.

Figure 2-1 The sketch of a curvilinear triangular element

（a）The curvilinear triangle in the coordinate system; (b) The curvilinear triangle in the coordinate system

Using the following coordinate transformation, the curvilinear triangle in the rectangular coordinate system, as shown in Figure 2-1(a), can be mapped onto the triangle defined in a parametric coordinate system, as shown in Figure 2-1(b)

(2-1)

where  denote the rectangular coordinates of the six controlling nodes in Figure 2-1a, ，，are the parametric coordinates varying from 0 to 1, and they satisfy the relation

(2-2)

From (2-2), it is clear that only two variables out of these three are independent.

…

- 2.3.3 The Choice of Basis Functions

After the geometrical discretization of the object surface using planar or curvilinear triangular elements, basis functions can be defined on these triangular elements to expand the unknown vector functions.

…

- 2.3.3.1 Planar RWG Basis Functions

Introduced by Rao Wilton, and Glisson in 1982, the RWG basis function [6] is defined over two adjacent triangular elements.

…

- 2.3.3.2 Curvilinear RWG Basis Functions

In order to give a better representation of curved surfaces, the curvilinear triangular elements can be used. Correspondingly, the curvilinear RWG basis functions [7] can be defined on the curvilinear triangular elements.

…

- 2.3.4 The Solution of Matrix Equations

The matrix equation can be solved with two types of algorithms, the direct algorithms and the iterative algorithms. They will be introduced briefly in this subsection [8].

…

- 2.3.4.1 Direct Algorithms

The commonly used direct algorithms include the Gaussian elimination, the LU decomposition, and the singular value decomposition (SVD) [9-10].

…

- 2.3.4.2 Iterative Algorithms

When the dimension of the impedance matrix is very large, the direct solution becomes very expensive.

…

- 2.4 Conclusion

…

- Chapter 3 Calderόn Preconditioner at Mid Frequencies

- 3.1 Introduction

The integral equations (IEs) are used to model the electromagnetic scattering,

…

- 3.2 Calderόn Relation and Calderόn Identities

In a scattering problem, according to the surface equivalence principle,

…

- 3.3 Calderόn Preconditioner at Mid Frequencies

Based on the discussion in the preceding section,

…

- 3.4 Numerical Examples

Two simple examples are given to demonstrate the fast convergence of the Calderόn preconditioner at mid frequencies.

…

- 3.5 Conclusion

The Calderόn preconditioner for the EFIE at mid frequencies is reviewed in this chapter.

…

- Chapter 4 Calderόn Preconditioning Technique for N-Müller

- 4.1 Introduction

Analysis of low-frequency electromagnetic problems has received more attention,

…

- 4.2 N-Müller Integral Equations

Consider the problem of electromagnetic wave scattering by a conducting surface,

…

Theorem 1 …

Proof:

Consider the problem of electromagnetic wave scattering by a conducting surface,

…

the problem is proved.                                                  ■

- 4.3 The Derivation of N-Müller Equations

The derivation begins from the preconditioning of the EFIE,

…

- 4.4 The Discretization of N-Müller Equations

The derivation begins from the preconditioning of the EFIE,

…

- 4.5 Numerical Examples

In this section, the performance of the N-M¨uller equations is investigated.

…

- 4.6 Conclusion

In this chapter,

…

- Chapter 5 Conclusions

- 5.1 Concluding Remarks

The accurate and efficient numerical solutions of the Maxwell’s equations have important significance to the analysis of electromagnetic scattering and radiation problems.

…

- 5.2 Future Work

The researches reported in this dissertation have covered most important areas including the convergence acceleration of the first-kind integral equations and the accuracy improvement of the second-kind integral equations for both the PEC and the dielectric cases. Nevertheless, due to the time limitation, there are still spaces for the future development of the Calder´on-technique-related methods.

…

Acknowledgements

On the completion of this thesis,

…

References

- W. C. Chew, J. M. Jin, E. Michielssen, et al. Fast and Efficient Algorithms in Computational Electromagnetics[M]. Boston: Artech House, 2000

- 盛新庆.计算电磁学要论[M].北京:科学出版社, 2004

- 王秉中.计算电磁学[M].北京:科学出版社, 2001

- 吕英华.计算电磁学的数值方法[M].北京:清华大学出版社, 2006

- 王长清.现代计算电磁学基础[M].北京:北京大学出版社, 2005

- 潘小敏.计算电磁学中的并行技术及其应用[D]. 北京:中国科学院电子学研究所, 2006

- 中华人民共和国国家技术监督局.GB3100-3102. 中华人民共和国国家标准--量与单位[S]. 北京:中国标准出版社, 1994年11月1日

- W. C. Gibson. The Method of Moments in Electromagnetics[M]. New York: Chapman and Hall/CRC, 2008

- 胡俊.复杂目标矢量电磁散射的高效算法——快速多极子方法及其应用[D].成都:电子科技大学, 2000

- H. C. Martin, G. F. Carey. Introduction to Finite Element Analysis: Theory and Application [M]. New York: McGraw Hill, 1973

Research Results Obtained During the Study for Master Degree

- S. Wang, S. R Zhang, X. H. Zhou, et al. Effect of sintering atmospheres on the microstructure and dielectric properties of Yb/Mg Co-doped BaTiO3 ceramics[J]. Mater. Lett.  2005, 59: 2457-2460

- S. Wang, S. R Zhang, X. H. Zhou, et al. Effects of complex dopants on the microstructure and dielectric properties of BCTZ ceramics[J]. J. Mater. Sci. Mater. Electr. 2005,16(5):257-261

- S. Wang, S. R Zhang, X. H. Zhou, et al. Influence of sintering atmosphere on the microstructure and electrical properties of BaTiO3-based X8R materials[J]. J. Mater. Sci., Accepted

- 王升,周晓华,张树人,等.复合氧化物掺杂法制备抗还原BCTZ系陶瓷的研究[J]. 无机材料学报,2005, 20(1): 119-125

- Y. Zhang, L. M. Li, B. Li. Network traffic modeling using fully-stable cascades[C]. 2005 International Conference on Communication, Circuits and Systems(ICCCAS’05), Hong Kong, 2005, 2: 726-730

- 王升(4).XXX的陶瓷研究.四川省科技进步三等奖, 2003年12月

- 任智,郭伟.基于负载均衡的自组网并行多经路由方法[P].中国,发明专利,200510021537.7, 2005年8月